Question: What if two devices have one MAC address

An adaptive Ethernet bridge connects two Ethernets, forwards frames from one to the other, and uses  source addresses in packets to learn which computers are on which Ethernet. A bridge uses the location of computers to eliminate unnecessary forwarding

a bridge can only connect two networks that use the same technology, but a router can connect arbitrary networks

Routers use the destination network, not the destination computer, when forwarding a packet

TCP/IP protocols place much of the intelligence in hosts — routers in the Internet forward Internet packets, but do not participate in higher-layer services

The Original IPv4 Classful Addressing Scheme

slash notation

Classless Inter-Domain Routing (CIDR)

CIDR notation

128.211.168.0 / 21

Classless IPv4 addressing, which is now used throughout the Internet, assigns each ISP a CIDR block and allows  the ISP to partition addresses into contiguous subblocks, where the lowest address in a subblock starts at a power of two and the subblock contains a power of two addresses

private addresses or nonroutable addresses.

conventional computer that has two or more physical network connections (such computers are known as multi-homed hosts)

IPv4 Network Address

An IPv4 address that has a host ID of 0 is reserved to refer to the network

IPv4 Directed Broadcast Address

IPv4 supports directed broadcast in which a packet is sent to all computers on a specific network; hardware broadcast is used if available. A directed broadcast address has a valid network portion and a hosted of all 1s.

To avoid potential problems, many sites configure routers to reject all directed broadcast packets

IPv4 Limited (Local Network) Broadcast Address

An IPv4 limited broadcast address consists of thirty-two 1 bits. A packet sent to the limited broadcast address will be broadcast across the local network, and can be used at startup before a computer learns its IP address

IPv4 All-0s Source Address

In IPv4, an address with thirty-two 0 bits is used as a temporary source address at startup before a host learns its IP address.

IPv4 Loopback Address

IPv4 reserves 127.0.0.0 / 8 for loopback testing; a packet destined to any host with prefix 127 stays within the computer and does not travel across a network

Weaknesses In Internet Addressing

If a host computer moves from one network to another, its internet address must change

Internet Address Assignment And Delegation Of Authority

Internet Assigned Numbers Authority (IANA)

Internet Corporation for Assigned Names and Numbers (ICANN)

that ICANN has authorized to administer blocks of addresses (ARIN, RIPE, APNIC, LACNIC, or AFRINIC).

The Address Resolution Protocol, ARP, allows a host to find the physical address of a target host on the same physical network, given only the target’s IP address.

The IP Datagram

IPv4 Datagram Format

IPv6 Datagram Format

IPv6 Base Header Format

maximum transfer unit, maximum transmission unit or MTU

IPv6 Fragmentation And Path MTU Discovery (PMTUD)

Asynchronous Transfer Mode (ATM)

Path MTU Discovery (PMTUD),

In the Internet, the ultimate destination reassembles fragments. The design means that routers do not need to store fragments or keep other information about packets

Time To Live (IPv4) And Hop Limit (IPv6)

IP software in each machine along a path from source to destination decrements the field known as TIME-TO-LIVE (IPv4) or HOP LIMIT (IPv6). When the field reaches zero the datagram is discarded.

IPv4 Options

Forwarding Information Base (FIB).

The Internet design distinguishes between hosts and routers. Although a host with multiple network connections can be configured to act as a router, the resulting system may not perform as expected.

Direct And Indirect Delivery

Transmission of an IP datagram between two machines on a single physical network does not involve routers. The sender encapsulates the datagram in a physical frame, binds the next-hop address to a physical hardware address, and sends the resulting frame directly to the destination.

Indirect Delivery

Routers in a TCP/IP internet form a cooperative, interconnected structure. Datagrams pass from router to router until they reach a router that can deliver the datagram directly.

- A host only knows about directly-connected networks; a host relies on routers to transfer datagrams to remote destinations.

- Each router knows how to reach all possible destinations in the internet;given a datagram, a router can forward it correctly.

Because it allows forwarding to be based on network prefixes, the IP addressing scheme controls the size of forwarding tables.

Next-Hop Forwarding

To hide information, keep forwarding tables small, and make forwarding decisions efficient, IP forwarding software only keeps information about destination network addresses, not about individual host addresses.

ICMP Message Delivery

The User Datagram Protocol

The User Datagram Protocol (UDP) provides an unreliable, best-effort, connectionless delivery service using IP to transport messages between machines. UDP uses IP to carry messages, but adds the ability to distinguish among multiple destinations within a given host computer.

UDP Message Format

The IP layer is responsible only for transferring data between a pair of hosts on an internet, while the UDP layer is responsible only for differentiating among multiple sources or destinations within one host.

Properties Of The Reliable Delivery Service

- Stream Orientation

- Virtual Circuit Connection

- Buffered Transfer

- Unstructured Stream

- Full Duplex Communication

For applications where data must be transferred without waiting to fill a buffer, the stream service provides a push mechanism that applications use to force immediate transfer.

Reliability: Acknowledgements And Retransmission

positive acknowledgement with retransmission (PAR)

send-and-wait

The Sliding Window Paradigm

Layering, Ports, Connections, And Endpoints

TCP uses the connection, not the protocol port, as its fundamental abstraction; connections are identified by a pair of endpoints.

Because TCP identifies a connection by a pair of endpoints, a given TCP port number can be shared by multiple connections on the same machine.

Passive And Active Opens

Variable Window Size And Flow Control

A TCP acknowledgement specifies the sequence number of the next octet that the receiver expects to receive.

To accommodate the varying delays encountered in an internet environment, TCP uses an adaptive retransmission algorithm that monitors delays on each connection and adjusts its timeout parameter accordingly.

Tail-Drop Policy For Routers: if a packet queue is filled when a datagram must be placed on the queue, discard the datagram.

Random Early Detection (RED)

RED Policy For Routers: if the input queue is full when a datagram arrives, discard the datagram; if the input queue is below a minimum threshold, add the datagram to the queue; otherwise, discard the datagram with a probability that depends on the queue size.

Establishing A TCP Connection

Closing a TCP Connection

TCP State Machine

241