Question: What if two devices have one MAC address

An adaptive Ethernet bridge connects two Ethernets, forwards frames from one to the other, and uses  source addresses in packets to learn which computers are on which Ethernet. A bridge uses the location of computers to eliminate unnecessary forwarding

a bridge can only connect two networks that use the same technology, but a router can connect arbitrary networks

Routers use the destination network, not the destination computer, when forwarding a packet

TCP/IP protocols place much of the intelligence in hosts — routers in the Internet forward Internet packets, but do not participate in higher-layer services

The Original IPv4 Classful Addressing Scheme

slash notation

Classless Inter-Domain Routing (CIDR)

CIDR notation

128.211.168.0 / 21

Classless IPv4 addressing, which is now used throughout the Internet, assigns each ISP a CIDR block and allows  the ISP to partition addresses into contiguous subblocks, where the lowest address in a subblock starts at a power of two and the subblock contains a power of two addresses

private addresses or nonroutable addresses.

conventional computer that has two or more physical network connections (such computers are known as multi-homed hosts)

IPv4 Network Address

An IPv4 address that has a host ID of 0 is reserved to refer to the network

IPv4 Directed Broadcast Address

IPv4 supports directed broadcast in which a packet is sent to all computers on a specific network; hardware broadcast is used if available. A directed broadcast address has a valid network portion and a hosted of all 1s.

To avoid potential problems, many sites configure routers to reject all directed broadcast packets

IPv4 Limited (Local Network) Broadcast Address

An IPv4 limited broadcast address consists of thirty-two 1 bits. A packet sent to the limited broadcast address will be broadcast across the local network, and can be used at startup before a computer learns its IP address

IPv4 All-0s Source Address

In IPv4, an address with thirty-two 0 bits is used as a temporary source address at startup before a host learns its IP address.

IPv4 Loopback Address

IPv4 reserves 127.0.0.0 / 8 for loopback testing; a packet destined to any host with prefix 127 stays within the computer and does not travel across a network

Weaknesses In Internet Addressing

If a host computer moves from one network to another, its internet address must change

Internet Address Assignment And Delegation Of Authority

Internet Assigned Numbers Authority (IANA)

Internet Corporation for Assigned Names and Numbers (ICANN)

that ICANN has authorized to administer blocks of addresses (ARIN, RIPE, APNIC, LACNIC, or AFRINIC).

The Address Resolution Protocol, ARP, allows a host to find the physical address of a target host on the same physical network, given only the target’s IP address.

109